TITLE AND DATE:

Document name: Project #2 Cyclic Executive

Document reference: patel,neev.cmpe311.fall25.project#2

Date of publication: November 11, 2025

LEAD ENGINEER:

Neev Patel, CMPE311, University of Maryland Baltimore County (UMBC)

STAKEHOLDERS:

Prof Kidd, Instructor, University of Maryland Baltimore County, Baltimore, Maryland, USA MD Safwan Zaman, TA, University of Maryland Baltimore County, Baltimore, Maryland, USA

HIGH-LEVEL DESCRIPTION:

This document specifies, designs, and validates an embedded system on an Arduino Uno R3 that performs identically to Project #1 Asynchronous Execution but uses a Cyclic Executive (CE) task manager instead of asynchronous timing calls. The CE executes a set of small, non-blocking tasks (serial input, LED1 blink, LED2 blink) in a round-robin order through a function-pointer array. Each task runs once per short frame (5 ms), ensuring both LEDs continue blinking independently while serial input is processed.

DESCRIPTION:

The problem is to reproduce the same dual-LED blinker from Project #1 while demonstrating a polled, round-robin scheduler. The user selects either LED 1 or LED 2 and provides a blink interval in milliseconds. The chosen LED blinks with a 50 % duty cycle at that interval while the other LED continues unchanged.
The system runs entirely on the Arduino Uno R3, uses the Serial Monitor for I/O, and contains no blocking code—each task returns quickly so the cyclic executive can keep cycling on schedule.

REFERENCES:

patel,neev.cmpe311.fall25.project#1 – Project #1 Asynchronous Execution, Sept 25 2025

Course lecture slides on Cyclic Executives and Function Pointer Scheduling

DEFINITIONS AND ABBREVIATIONS:

“The User” – The person operating (not programming) the embedded system
“The System” – The embedded system being operated by The User
“The Customer” – The person(s) paying for the embedded system being designed and built
“The Developer” – The person(s) designing and building the System
“The Evaluator” – The person(s) that determine whether or not The System satisfies The
Customer-requirements.
“The Customer-requirements” – The requirements defined by The Customer as satisfying The
Contract.
“The Requirements” – The System’s high-level technical requirements derived from The
Customer-requirements.
“The Educational-constraints” – Requirements imposed by the instructor unrelated to the
embedded system that allow The System to be evaluated.
“The Company” – The organization The Customer has contracted with to build The System.
“The Contract” – The business document that legally binds The Company to provide some
service or product to The Customer.
“serial-monitor” – The serial port used by the Arduino IDE to communicate with The User.
“The Reference-platform” – The configuration of The System used by The Developer to test and
validate The System. For this class, The System is the Arduino compatible ELEGOO Uno R3
development board.

Arduino – an Italian open-source hardware and software company; also refers to a
development board created by the company
arduino.h – header for a library of convenience functions specific to the Arduino development
platform
AVR – A family of microcontrollers, originally developed by Atmel, and currently owned by
Microchip Technology
ELEGOO – A Chinese company that develops and markets 3D printers and accessories
IDE – Integrated Development Environment
gcc – front end for the GNU Compiler Collection
Github – A widely used distributed SVC (Software Version Control) system
LED – Light Emitting Diode

Cyclic Executive (CE): A scheduling method that repeatedly runs a fixed list of non-blocking tasks in a known order once every frame period.

CONVENTIONS:

Must, shall or will – your design must satisfy the requirement

May – your design may satisfy the requirement but doesn’t have to

Informative – the intent of the following description is to make the requirement more understandable

All customer requirements are started with “C.#”. All high-level requirements are started with “HL.#”.

All testing/validation requirements ar5e started with “T.#”

CUSTOMER REQUIREMENTS: <list the customer requirements which I gave you>

  - The User must be able to set the blink rate of two different LEDs

    - The User  must be able to update the blink rate of each of the LEDs independently

  - The LED must blink at the set rate until the User   tells the LED to blink at a different rate.

  - The System must run upon an Arduino UNO R3 compatible development board.

HIGH-LEVEL TECHNICAL REQUIREMENTS: <list the high-level technical requirements I gave you>

HL.1 The System must use at least 2 LEDs
HL.2 The System must use a standard Arduino compatible development board (e.g. the
provided ELEGOO Uno R3)
HL.3 The System must communicate with The User only via the Arduino IDE serial-
monitor port
HL.4 Any use of the serial-monitor in HL.3 must be asynchronous and not affect the
blinking of the LEDs.
HL.5 The User must be able to set the blink interval of the LEDs in msec
HL.6 The blink rate of each LED must be constant unless changed by The User

TESTING AND VALIDATION REQUIREMENTS:

| Serial Port I/O | Notes |
| --- | --- |
| What LED? (1 or 2) 1 |  |
| What interval (in msec)? 1600 | LED 1 starts blinking on and off at 300ms on and off rate while LED2 isnt affected |
| What LED? (1 or 2) 2 |  |
| What interval (in msec)? 600 | LED 2 starts blinkings on and off at 800ms on and off rate while LED1 isnt affect |
| What LED? (1 or 2) | Waiting for the next LED# interval pair |
| What LED? (1 or 2) 1 |  |
| What interval (in msec)? 1600 | LED 1 starts blinking on and off at 800ms on and off rate while LED2 isnt affected |
| What LED? (1 or 2) 2 |  |
| What interval (in msec)? 600 | LED 2 starts blinkings on and off at 200ms on and off rate while LED 1 isnt affect |
| What LED? (1 or 2) | Waiting for the next LED# interval pair |

Table 1: Shows expected output for what should happen

T.1: The series of command should work as it does in Table 1

T.2: The LED blink rate shouldn’t change once we provide the blink rate to the LED until we changes it ourselves

T.3: blink rate must be in milliseconds

T.4 Whatever blink rate is chosen must be properly displayed onto the breadboard LEDs

DESIGN:

Equipment:

- EQUIPMENT REQUIRED:
• Arduino Uno R3 development board or clone, e.g. the ELEGOO Uno R3
• LEDs, qty 2
• 200W ¼ Watt resistors, qty 2
• 10kW ¼ Watt resistor, qty 1
• Arduino IDE 2.3.3 or better
• Laptop or other computer to run the Arduino IDE
• Cable, USB 2.0 female type B to female USB 2.0 type A

Circuit Explanation:

Same wiring as Project #1: pins 8 and 9 each connect to an LED in series with a resistor, both grounded to the Arduino GND rail.

CODE:

const int led1 = 8;

const int led2 = 9;

unsigned long interval1 = 0;

unsigned long interval2 = 0;

unsigned long preMillis1 = 0;

unsigned long preMillis2 = 0;

bool ledState1 = LOW;

bool ledState2 = LOW;

int selectedLED = 0;

int inputStage = 0;

String myInput = "";

const unsigned long FRAME_MS = 5;

void taskSerial();

void taskLed1();

void taskLed2();

void (*tasks[])() = { taskSerial, taskLed1, taskLed2 };

const int NUM_TASKS = 3;

void setup() {

pinMode(led1, OUTPUT);

pinMode(led2, OUTPUT);

Serial.begin(9600);

Serial.println("What LED? (1 or 2)");

}

void loop() {

unsigned long frameStart = millis();

for(int i = 0; i < NUM_TASKS; i++){

tasks[i]();

}

unsigned long elapsed = millis() - frameStart;

if(elapsed < FRAME_MS){

delay(FRAME_MS - elapsed);

}

}

void taskSerial(){

while (Serial.available()) {

char c = Serial.read();

if (c == '\r' || c == '\n') {

if (myInput.length() > 0) {

if (inputStage == 0) {

selectedLED = myInput.toInt();

if (selectedLED == 1 || selectedLED == 2) {

Serial.println("What interval (in msec)?");

inputStage = 1;

} else {

Serial.println("Invalid LED. Choose 1 or 2");

}

} else if (inputStage == 1) {

int interval = myInput.toInt();

if (interval > 0) {

if (selectedLED == 1) {

interval1 = interval;

} else {

interval2 = interval;

}

} else {

Serial.println("Invalid interval");

}

Serial.println("What LED? (1 or 2)");

inputStage = 0;

}

myInput = "";

}

} else {

myInput += c;

}

}

}

void taskLed1(){

if (interval1 == 0){

return;

}

unsigned long now = millis();

if(now - preMillis1 >= interval1){

preMillis1 = now;

ledState1 = !ledState1;

digitalWrite(led1, ledState1);

}

}

void taskLed2(){

if (interval2 == 0){

return;

}

unsigned long now = millis();

if(now - preMillis2 >= interval2){

preMillis2 = now;

ledState2 = !ledState2;

digitalWrite(led2, ledState2);

}

}

Code Explanation:

The program is divides the system into three short, non-bocking tasks: taskSerial, taskLed1, and taskLed2. These are stored In small array of function pointers and are called once per frame by the main loop() function. Each LED task checks its timer and toggles when its interval expires, while the serial task handles user commands. Because every task runs quickly and the frame repeats every 5ms, both LEDs blink independently while user input remains responsive.

TESTING:

View my video for the results of the test.

Link provides the video:

https://drive.google.com/file/d/1jpcf3xFFdownNbbE5L_EoMBehF-T-5mA/view?usp=drive_link

END OF DOCUMENT