TITLE AND DATE:

Document name: Project #5 FreeRTOS

Document reference: patel,neev.cmpe311.fall25.project#5

Date of publication: December 10, 2025

LEAD ENGINEER:

Neev Patel, CMPE311, University of Maryland Baltimore County (UMBC)

STAKEHOLDERS:

Prof Kidd, Instructor, University of Maryland Baltimore County, Baltimore, Maryland, USA MD Safwan Zaman, TA, University of Maryland Baltimore County, Baltimore, Maryland, USA

HIGH-LEVEL DESCRIPTION:

This document specifies, designs, and validates Project #5, which re-implements Project #4 (PWM motor control) using FreeRTOS task management APIs instead of a Cyclic Executive (CE). The System behavior remains functionally identical to the previous project while replacing all CE scheduling logic with FreeRTOS tasks, delays, and synchronization mechanisms. The System allows The User to vary LED brightness or motor speed in five discrete steps using a physical pushbutton. Each press advances the PWM duty cycle from OFF → minimum → maximum → OFF, while all tasks execute asynchronously and non-blocking under FreeRTOS control.

This project fulfills the instructional objective of exposing students to RTOS-based task scheduling, while preserving the operational behavior, timing, and safety constraints of the prior PWM motor-control project

DESCRIPTION:

The System allows The User to control the speed of a DC motor or the brightness of an LED using a pushbutton. Each press advances the PWM duty cycle through five discrete levels from OFF to maximum intensity, then wraps back to OFF. Button input is debounced in software to ensure reliable operation.

PWM is generated in software using Timer1 interrupts, ensuring precise and stable timing independent of FreeRTOS task execution. FreeRTOS is used exclusively for task management, replacing all Cyclic Executive scheduling logic from previous projects.

REFERENCES:

Project specification: Project Add Duty Cycle, course PDF (2025)

projectAddDutyCycle (1)

Arduino documentation for PWM and pinMode
Project #3 Cyclic Executive
ELEGOO Uno R3 hardware reference

DEFINITIONS AND ABBREVIATIONS:

All definitions follow Projects #1–#3 and include:

“The User” – The person pressing the pushbutton
“The System” – The embedded CE-based PWM motor/LED controller
“The Customer” – The party requesting the embedded system
“The Developer” – The person designing (Neev Patel)
“The Evaluator” – Instructor/TA verifying requirements
“CE” – Cyclic Executive
“PWM” – Pulse Width Modulation
TON / TOFF – HIGH and LOW durations of one PWM cycle
ISR – Interrupt Service Routine
Duty Cycle – Percentage of time signal is HIGH in one period

CONVENTIONS:

Must, shall or will – your design must satisfy the requirement

May – your design may satisfy the requirement but doesn’t have to

Informative – the intent of the following description is to make the requirement more understandable

All customer requirements are started with “C.#”. All high-level requirements are started with “HL.#”.

All testing/validation requirements ar5e started with “T.#”

CUSTOMER REQUIREMENTS: <list the customer requirements which I gave you>

C.1 The User must be able to change the LED/motor duty cycle using a pushbutton.
C.2 The System must support at least 5 distinct duty-cycle states.
C.3 The LED/motor brightness must remain stable until The User changes it.
C.4 The project must build upon the CE scheduler of Project #3.
C.5 The System must run on an Arduino Uno R3.

HIGH-LEVEL TECHNICAL REQUIREMENTS: <list the high-level technical requirements I gave you>

HL.1 The System must generate PWM in software using TON and TOFF counters.
HL.2 The CE must use a 5 ms frame.
HL.3 The PWM duty cycle must update without blocking the CE.
HL.4 The System must debounce the button in software.
HL.5 Duty cycle increments must occur in fixed STEP sizes, wrapping at maximum.
HL.6 The System must use a MOSFET low-side driver circuit for the motor/LED load.
HL.7 A flyback diode must be used across the motor terminals.
HL.8 All tasks must run in constant worst-case time so as not to violate frame constraints.

TESTING AND VALIDATION REQUIREMENTS:

| Button Press | Duty Cycle (%) | TON | TOFF | Notes |
| --- | --- | --- | --- | --- |
| 1 | 0% | 0 | PERIOD | LED OFF / motor off |
| 2 | 25% | 1 | 3 | Visible dim/slow |
| 3 | 50% | 2 | 2 | Medium brightness/speed |
| 4 | 75% | 3 | 1 | High brightness/speed |
| 5 | 100% | PERIOD | 0 | Fully ON |
| 6 | wraps to 0% | 0 | PERIOD | Cycle restarts |

Table 1: Shows expected output for what should happen

T.1 Pressing the button cycles through duty cycles {0%, 25%, 50%, 75%, 100%}.
T.2 Each duty cycle produces visibly different brightness/motor speed.
T.3 PWM period remains stable and repeats every TON + TOFF frames.
T.4 PWM HIGH and LOW times match the calculated TON/TOFF values.
T.5 The CE does not miss frames or drift timing.
T.6 Debouncing prevents multiple increments from one physical press.
T.7 The MOSFET correctly switches and remains within safe temperature.

DESIGN:

Equipment:

- EQUIPMENT REQUIRED:
• Arduino Uno R3 development board or clone, e.g. the ELEGOO Uno R3
• N-channel logic-level MOSFET
• DC motor or LED load
• Flyback diode
• Pushbutton, qty 1
• 10kohm resistor (button pulldown)
• 1kohm gate resistor, 4xAA 6V battery pack. Breadboard + jumpers

Circuit Explanation:

PWM output path

Pin 9 → 1 kΩ → MOSFET Gate
Gate pulled down to GND through 10 kΩ
MOSFET Drain → Motor –
Motor + → 6 V battery pack +
MOSFET Source → GND (shared with Arduino)
Flyback diode across motor terminals

Button input

Button → Arduino pin 2 
10 kΩ pulldown → GND
Button → +5 V when pressed

CODE:

#include <Arduino.h>

#include <Arduino_FreeRTOS.h>

const uint8_t PWM_PIN=9;

const uint8_t BUTTON_PIN=2;

const uint8_t LED2_PIN=12;

volatile uint16_t pwmCounter=0;

volatile uint16_t pwmTop=800;

volatile uint16_t pwmDuty=0;

const uint16_t dutyTable[9]={0,200,400,600,800,600,400,200,0};

const char* dutyDesc[9]={

"motor off",

"motor at 2/8 duty",

"motor at 4/8 duty",

"motor at 6/8 duty",

"motor at 8/8 duty",

"motor at 6/8 duty",

"motor at 4/8 duty",

"motor at 2/8 duty",

"motor off"

};

volatile uint8_t dutyIndex=0;

bool lastStableState=LOW;

bool debouncedState=LOW;

unsigned long lastDebounceTimeMs=0;

const unsigned long debounceDelayMs=25;

volatile bool blinkOn=true;

void TaskBlinkPWM(void *pvParameters);

void TaskBlinkLED2(void *pvParameters);

void TaskButton(void *pvParameters);

void TaskUpdatePWMDuty(void *pvParameters);

void setupTimer1ForPWM(){

noInterrupts();

TCCR1A=0;

TCCR1B=0;

TCCR1B|=(1<<WGM12);

OCR1A=99;

TIMSK1|=(1<<OCIE1A);

TCCR1B|=(1<<CS11)|(1<<CS10);

interrupts();

pinMode(PWM_PIN,OUTPUT);

}

ISR(TIMER1_COMPA_vect){

pwmCounter++;

if(pwmCounter>=pwmTop) pwmCounter=0;

if(blinkOn && (pwmCounter<pwmDuty)){

digitalWrite(PWM_PIN,HIGH);

}else{

digitalWrite(PWM_PIN,LOW);

}

}

void setup(){

pinMode(PWM_PIN,OUTPUT);

pinMode(LED2_PIN,OUTPUT);

pinMode(BUTTON_PIN,INPUT);

Serial.begin(9600);

Serial.println("Project #5 FreeRTOS PWM + Blink on Pin 9");

setupTimer1ForPWM();

dutyIndex=0;

pwmDuty=dutyTable[dutyIndex];

Serial.println("Initial state: motor off.");

xTaskCreate(TaskBlinkPWM,"BlinkPWM",128,NULL,1,NULL);

xTaskCreate(TaskBlinkLED2,"Blink2",128,NULL,1,NULL);

xTaskCreate(TaskButton,"Button",256,NULL,2,NULL);

xTaskCreate(TaskUpdatePWMDuty,"PWMupd",128,NULL,2,NULL);

vTaskStartScheduler();

}

void loop(){}

void TaskBlinkPWM(void *pvParameters){

while(1){

blinkOn=true;

vTaskDelay(pdMS_TO_TICKS(900));

blinkOn=false;

vTaskDelay(pdMS_TO_TICKS(100));

}

}

void TaskBlinkLED2(void *pvParameters){

bool state=false;

while(1){

state=!state;

digitalWrite(LED2_PIN,state);

vTaskDelay(pdMS_TO_TICKS(700));

}

}

void TaskButton(void *pvParameters){

while(1){

int raw=digitalRead(BUTTON_PIN);

if(raw!=lastStableState){

lastStableState=raw;

lastDebounceTimeMs=millis();

}

if((millis()-lastDebounceTimeMs)>debounceDelayMs){

if(debouncedState!=raw){

debouncedState=raw;

if(debouncedState==HIGH){

dutyIndex=(dutyIndex+1)%9;

Serial.print("Duty: ");

Serial.print(dutyDesc[dutyIndex]);

Serial.print("  Index: ");

Serial.println(dutyIndex);

}

}

}

vTaskDelay(pdMS_TO_TICKS(5));

}

}

void TaskUpdatePWMDuty(void *pvParameters){

while(1){

pwmDuty=dutyTable[dutyIndex];

vTaskDelay(pdMS_TO_TICKS(20));

}

}

Code Explanation:

This program demonstrates PWM motor control using FreeRTOS multitasking and a Timer1 interrupt on the Arduino. Timer1 is configured in CTC mode to trigger an ISR at ~2.5 kHz, where software-based PWM is generated by comparing a counter to a duty value. The duty cycle is selected from a lookup table and changed using a debounced pushbutton. FreeRTOS runs four tasks in parallel: one enables/disables PWM to create a blink effect, one blinks an LED on pin 12, one reads and debounces the button, and one updates the PWM duty value. This allows the system to adjust motor speed while simultaneously blinking an LED and maintaining stable PWM output without blocking delays, demonstrating the benefits of real-time multitasking.

TESTING:

View my video for the results of the test.

Link provides the video:

https://drive.google.com/file/d/18ZJ6mzmNLbi-zkN2LPwtG4qmVEx0j2-l/view?usp=sharing

END OF DOCUMENT