TITLE AND DATE:

Document name: Project #3 PWM Motor Driver

Document reference: patel,neev.cmpe311.fall25.project#3

Date of publication: December 2, 2025

LEAD ENGINEER:

Neev Patel, CMPE311, University of Maryland Baltimore County (UMBC)

STAKEHOLDERS:

Prof Kidd, Instructor, University of Maryland Baltimore County, Baltimore, Maryland, USA MD Safwan Zaman, TA, University of Maryland Baltimore County, Baltimore, Maryland, USA

HIGH-LEVEL DESCRIPTION:

This document specifies, designs, and validates an embedded system on an Arduino Uno R3 that extends the Cyclic Executive (CE) scheduler from Project #3 with a new feature: the ability for The User to modify PWM duty cycle in real time using a physical pushbutton. The LED brightness (or motor speed, depending on configuration) is increased in discrete steps each time the button is pressed. The System safely maintains all timing constraints while integrating PWM generation, debouncing, MOSFET motor driving, and CE-based task scheduling.

The project’s design is divided into three required parts from the official assignment specification:
Part I: Software PWM generation using TON/TOFF frame timing.
Part II: MOSFET-based motor driver and diode protection circuit.
Part III: Integration of PWM and button task into the CE used in Project #3.

The System is fully non-blocking, satisfies all scheduling constraints, and meets the Customer, Technical, and Educational Requirements defined in the assignment.

DESCRIPTION:

The goal is to add PWM control to the existing CE scheduler so that The User can adjust brightness/speed via a debounced button. Each press increases duty cycle by a fixed increment (STEP), cycling through 0%, 25%, 50%, 75%, and 100%. The PWM is generated entirely in software using two counters (TON and TOFF), and the CE ensures the PWM task runs every 5 ms frame without drift.

The System uses a MOSFET low-side switching circuit as described in the project specification. A flyback diode prevents MOSFET damage during inductive current changes. All tasks return quickly to maintain timing integrity.

This project demonstrates integration of real-time scheduling, hardware-safe motor driving, PWM waveform generation, and user input processing.

REFERENCES:

Project specification: Project Add Duty Cycle, course PDF (2025)

projectAddDutyCycle (1)

Arduino documentation for PWM and pinMode
Project #3 Cyclic Executive
ELEGOO Uno R3 hardware reference

DEFINITIONS AND ABBREVIATIONS:

All definitions follow Projects #1–#3 and include:

“The User” – The person pressing the pushbutton
“The System” – The embedded CE-based PWM motor/LED controller
“The Customer” – The party requesting the embedded system
“The Developer” – The person designing (Neev Patel)
“The Evaluator” – Instructor/TA verifying requirements
“CE” – Cyclic Executive
“PWM” – Pulse Width Modulation
TON / TOFF – HIGH and LOW durations of one PWM cycle
ISR – Interrupt Service Routine
Duty Cycle – Percentage of time signal is HIGH in one period

CONVENTIONS:

Must, shall or will – your design must satisfy the requirement

May – your design may satisfy the requirement but doesn’t have to

Informative – the intent of the following description is to make the requirement more understandable

All customer requirements are started with “C.#”. All high-level requirements are started with “HL.#”.

All testing/validation requirements ar5e started with “T.#”

CUSTOMER REQUIREMENTS: <list the customer requirements which I gave you>

C.1 The User must be able to change the LED/motor duty cycle using a pushbutton.
C.2 The System must support at least 5 distinct duty-cycle states.
C.3 The LED/motor brightness must remain stable until The User changes it.
C.4 The project must build upon the CE scheduler of Project #3.
C.5 The System must run on an Arduino Uno R3.

HIGH-LEVEL TECHNICAL REQUIREMENTS: <list the high-level technical requirements I gave you>

HL.1 The System must generate PWM in software using TON and TOFF counters.
HL.2 The CE must use a 5 ms frame.
HL.3 The PWM duty cycle must update without blocking the CE.
HL.4 The System must debounce the button in software.
HL.5 Duty cycle increments must occur in fixed STEP sizes, wrapping at maximum.
HL.6 The System must use a MOSFET low-side driver circuit for the motor/LED load.
HL.7 A flyback diode must be used across the motor terminals.
HL.8 All tasks must run in constant worst-case time so as not to violate frame constraints.

TESTING AND VALIDATION REQUIREMENTS:

| Button Press | Duty Cycle (%) | TON | TOFF | Notes |
| --- | --- | --- | --- | --- |
| 1 | 0% | 0 | PERIOD | LED OFF / motor off |
| 2 | 25% | 1 | 3 | Visible dim/slow |
| 3 | 50% | 2 | 2 | Medium brightness/speed |
| 4 | 75% | 3 | 1 | High brightness/speed |
| 5 | 100% | PERIOD | 0 | Fully ON |
| 6 | wraps to 0% | 0 | PERIOD | Cycle restarts |

Table 1: Shows expected output for what should happen

T.1 Pressing the button cycles through duty cycles {0%, 25%, 50%, 75%, 100%}.
T.2 Each duty cycle produces visibly different brightness/motor speed.
T.3 PWM period remains stable and repeats every TON + TOFF frames.
T.4 PWM HIGH and LOW times match the calculated TON/TOFF values.
T.5 The CE does not miss frames or drift timing.
T.6 Debouncing prevents multiple increments from one physical press.
T.7 The MOSFET correctly switches and remains within safe temperature.

DESIGN:

Equipment:

- EQUIPMENT REQUIRED:
• Arduino Uno R3 development board or clone, e.g. the ELEGOO Uno R3
• N-channel logic-level MOSFET
• DC motor or LED load
• Flyback diode
• Pushbutton, qty 1
• 10kohm resistor (button pulldown)
• 1kohm gate resistor, 4xAA 6V battery pack. Breadboard + jumpers

Circuit Explanation:

PWM output path

Pin 9 → 1 kΩ → MOSFET Gate
Gate pulled down to GND through 10 kΩ
MOSFET Drain → Motor –
Motor + → 6 V battery pack +
MOSFET Source → GND (shared with Arduino)
Flyback diode across motor terminals

Button input

Button → Arduino pin 2 
10 kΩ pulldown → GND
Button → +5 V when pressed

CODE:

#include <Arduino.h>

const uint8_t LED1_PIN   = 13;

const uint8_t LED2_PIN   = 12;

const uint8_t PWM_PIN    = 9;

const uint8_t BUTTON_PIN = 2;

typedef void (*TaskFn)();

struct Task {

TaskFn   fn;

uint16_t periodMs;

uint32_t lastRunTimeMs;

};

enum {

TASK_BLINK1 = 0,

TASK_BLINK2,

TASK_BUTTON,

TASK_PWM_UPDATE,

NUM_TASKS

};

Task tasks[NUM_TASKS];

volatile uint16_t pwmCounter = 0;

volatile uint16_t pwmTop     = 800;

volatile uint16_t pwmDuty    = 0;

const uint16_t dutyTable[9] = {0, 200, 400, 600, 800, 600, 400, 200, 0};

const char*    dutyDesc[9]  = {

"motor off",

"motor at 2/8 duty",

"motor at 4/8 duty",

"motor at 6/8 duty",

"motor at 8/8 duty",

"motor at 6/8 duty",

"motor at 4/8 duty",

"motor at 2/8 duty",

"motor off"

};

volatile uint8_t dutyIndex = 0;   // 0..8

bool lastStableState   = LOW;

bool debouncedState    = LOW;

unsigned long lastDebounceTime = 0;

const unsigned long debounceDelayMs = 25;

void setupTimer1ForPWM()

{

noInterrupts();

TCCR1A = 0;

TCCR1B = 0;

TCCR1B |= (1 << WGM12);

OCR1A = 99;

TIMSK1 |= (1 << OCIE1A);

TCCR1B |= (1 << CS11) | (1 << CS10);

interrupts();

pinMode(PWM_PIN, OUTPUT);

}

ISR(TIMER1_COMPA_vect)

{

pwmCounter++;

if (pwmCounter >= pwmTop) {

pwmCounter = 0;

}

if (pwmCounter < pwmDuty) {

digitalWrite(PWM_PIN, HIGH);

} else {

digitalWrite(PWM_PIN, LOW);

}

}

// Blink LED1 (fast)

void taskBlinkLED1()

{

static bool state = false;

state = !state;

digitalWrite(LED1_PIN, state ? HIGH : LOW);

}

// Blink LED2 (slow)

void taskBlinkLED2()

{

static bool state = false;

state = !state;

digitalWrite(LED2_PIN, state ? HIGH : LOW);

}

void taskButton()

{

int raw = digitalRead(BUTTON_PIN);

if (raw != lastStableState) {

lastStableState = raw;

lastDebounceTime = millis();

}

if ((millis() - lastDebounceTime) > debounceDelayMs) {

if (debouncedState != raw) {

debouncedState = raw;

if (debouncedState == HIGH) {

dutyIndex = (dutyIndex + 1) % 9;

Serial.print("Button press: ");

Serial.print(dutyDesc[dutyIndex]);

Serial.print(" (index ");

Serial.print(dutyIndex);

Serial.println(")");

}

}

}

}

void taskUpdatePWMDuty()

{

pwmDuty = dutyTable[dutyIndex];

}

void initTasks()

{

tasks[TASK_BLINK1].fn           = taskBlinkLED1;

tasks[TASK_BLINK1].periodMs     = 200;

tasks[TASK_BLINK1].lastRunTimeMs = 0;

tasks[TASK_BLINK2].fn           = taskBlinkLED2;

tasks[TASK_BLINK2].periodMs     = 700;

tasks[TASK_BLINK2].lastRunTimeMs = 0;

tasks[TASK_BUTTON].fn           = taskButton;

tasks[TASK_BUTTON].periodMs     = 5;

tasks[TASK_BUTTON].lastRunTimeMs = 0;

tasks[TASK_PWM_UPDATE].fn           = taskUpdatePWMDuty;

tasks[TASK_PWM_UPDATE].periodMs     = 20;

tasks[TASK_PWM_UPDATE].lastRunTimeMs = 0;

}

void runTasks()

{

uint32_t now = millis();

for (uint8_t i = 0; i < NUM_TASKS; i++) {

if (now - tasks[i].lastRunTimeMs >= tasks[i].periodMs) {

tasks[i].lastRunTimeMs = now;

tasks[i].fn();

}

}

}

void setup()

{

pinMode(LED1_PIN, OUTPUT);

pinMode(LED2_PIN, OUTPUT);

pinMode(BUTTON_PIN, INPUT);

Serial.begin(9600);

Serial.println("Project #4: PWM Motor Controller Starting...");

setupTimer1ForPWM();

initTasks();

dutyIndex = 0;

pwmDuty   = dutyTable[dutyIndex];

Serial.println("Initial state: motor off.");

}

void loop()

{

runTasks();

}

Code Explanation:

This Arduino program implements a simple cooperative task manager and a hand-built PWM motor controller using Timer1 interrupts. Timer1 is configured in CTC mode with a prescaler of 64 and an OCR1A value of 99, generating an interrupt at roughly 2.5 kHz. Inside this interrupt, a software PWM algorithm increments a counter from 0 to 800 and drives pin 9 HIGH or LOW depending on whether the counter is below the current duty value, creating a stable, adjustable PWM signal for a motor. In the foreground, the program runs four periodic tasks through a small task scheduler: two tasks blink LEDs at different rates, one task debounces a push button every 5 ms, and another updates the PWM duty cycle based on the current button state. The button cycles through nine predefined duty-cycle levels stored in a lookup table, and each valid button press rotates the motor through a pattern of speeds. The use of a non-blocking task manager ensures that LED blinking, button reading, and PWM updates occur concurrently without interfering with the precise timing of the PWM interrupt, demonstrating asynchronous operation and clean separation between real-time ISR work and lower-priority background tasks.

TESTING:

View my video for the results of the test.

Link provides the video:

https://drive.google.com/file/d/18ZJ6mzmNLbi-zkN2LPwtG4qmVEx0j2-l/view?usp=sharing

END OF DOCUMENT